### Dipstick - Data

/tmp/in/input.doc

Dipstick

# BN1 / 2

# Total length from tip to base of handle "loop":    450mm

# Length from tip to top of tube on block (ie length of stick inserted in tube/sump): 365mm

# Distance from tip to "low" mark: 77mm

# Distance from tip to "full" mark: 87mm

# Tube height 75mm measured from where tube comes out of block to highest part.

# BN4 / 6

BN7 / BT7

- 11 inches to full line and 12 inches to MIN level

- 1959 AH 3000 is 12-1/4 inches from tip to washer stop, with difference between min and max being 1inch.

# BJ7 / BJ8

- 17.25” and a hair

- 17 1/8” from tip to rubber washer (dust cover?).  ¼”  from tip to 'min' line and 1 5/16 to 'max' line; tube is 4 3/8” long

- measurement of dipstick for a BJ8 to bottom of the rubber washer is 17.125” inches

- stick measures 17-1/8" from bottom of rubber seal to end of the stick.

- length of the BJ8 dipstick washer to full line is 16" and 1" between full and low

- For BJ7 (and BJ8 engines up to serial number 29K/RU/H278) the dipstick measures about 1/16" shy of 11 inches from the "max" line to the bottom of the washer.  BJ8 engines 29K/RU/H279 and up are about 1/16" shy of 16 inches.

From:  /tmp/in/input.doc

Dipstick Tube

4 cyl tube is screwed in 		& 		6 cyl tube is pressed in

BN2 -   75mm measured from where tube comes out of the block to the highest part

BJ8  - 117mm measured from where tube comes out of the block to the highest part + 10mm into block = 127mm total length

From:   C:\British Cars\Austin Healey\Tech\A - Engine\Oil,Filters,etc\Dipstick Tube.doc