1965 Sunbeam Tiger B382000450LRXFE

Engine Build – 2004-2005

(289 Block - bore as found:  +0.040”)

*Block hot cleaned, measured and found to be an “early” thick-wall 289cu in

*Bored + .060” over and fitted with Keith Black forged pistons (9.76:1 C.R.)

Total Seal file-fit rings .060” over

*Crank:  SCAT 9000 Series Premium cast 3.000” stroke

*Rods: SCAT 4340 Forged I-Beam – 5.990” (bushed –Pins)

*Main and Rod Bearings: Clevite “P” Series

*Rotating Assembly Computer balanced to 28.2 oz.

* Melling stock oil pump.

*ARP Oil Pump Driveshaft

*Comp Cams Valve Train Assembly including:

MAGNUM 270 Roller cam – Duration 270° in and ex; .480” lift; Lobe Sep –110°

Roller hydraulic lifters with Valley Spider; Lifter installation kit (screw-in

Pedestals), push rods; 1.5 ratio roller-tipped rockers; duplex roller timing chain

And wheels; New cam bushings fitted and sized

*Nitrided Valves *Bronze Valve Guides

*Hi-performance valve springs with retainers and locks

*ARP fasteners used completely inside and out

*Head ported, matched with headers, and chambers equalized

*Flywheel re-surfaced and a Center-Force Clutch – 400hp model – fitted

*2008 – large alloy rad fitted with electric pusher fan, thermostatically controlled

EXTERNAL:

Edelbrock Performer 289 Intake manifold

Edelbrock 600 cfm 4 barrel carb

MSD 6AL Ignition module with MSD Coil & Tach module

MSD distributor

Period J.C. Whitney headers repaired and ceramic coated

Wilwood 4 piston calipers and 11” rotors on front wheels

Ford Toploader transmission rebuilt with David Kee parts,

Hurst competition shifter installed

Rear end ratio 3.07 (stock 2.88) Positraction unit installed

Interior by Sunbeam Specialties

Dave Munroe (mobile) 902-448-2250

dave@munroe.ca